- Install Driver

  - Portable Devices

While turn on the switch of the customized USB cable, there will be a Portable device, This driver will be auto installed by windows system.

  - mbed Serial Port

Power on the POS and connect the USB cable, there will be a mbed Serial Port.

  - Before install

  - After install

  - Start install

NOTE: Each USB port needs to install the driver once

- Choose the ScatterFile and Start

Decompress the download tool package：POS_SP_MDT(Topwise20210729).zip，Open the POS_MDT.exe.

- Connect and download

The download process consists of two steps:

- download boot image

- download other image

  - Download with boot

- Power off the POS;

- Use the customized USB cable, turn on the switch;

- POS connect to the USB;

- Turn off the switch

NOTE: The switch must be turned off before the boot download ends, since the boot download speed of the boot is very fast, you should turn off the switch when POS connect to the USB

- Start download and wait for download pass.

  - Download without boot image

1、Power off the POS;

2、Use the customized USB cable, turn off the switch;

3、POS connect to the USB;

4、Start download and wait for download pass.

- FAQ

  - How to block the message box of “Do you want to format it?”

So far, I have not found a way to stop the pop-up window, Please just ignore it.

  - the device always in the Portable devices mode, and unable to download boot

Please use the reset key to reset the pos